|  | Roll No. ___________ |  |
| --- | --- | --- |
| Pandit Deendayal Energy University, Gandhinagar |  |  |
| School of Technology |  |  |
| IA 1 Exam |  |  |
| Course Name : Computer Network |  | Max. Marks: 30 |

Part A: All questions are compulsory              [10X1= 10 Marks]

- If Bandwidth of an Ethernet can is 100Mbps, distance of the LAN is 1Km, velocity of signal in cable is 2*108 m/sec. Then what is minimum size of a frame in this Ethernet to detect collisions.

(a) 10,000 bits                 (b) 1000 bits             (c) 100 bits           (d) 1000 bytes

- If ‘K’ is the maximum number of bits available in sequence number field, then what is the maximum sender window size in GBN.

(a) 2K -1                              (b) 2K-1                   (c) 2K                                (d) 2K + 1

- If the distance between two nodes is 2 Km, velocity of signal in the medium is 2*108m/s, each frame is 1000 bits and bandwidth of the link is 1Gbps. If the channel is error free (no need of SR or GBN), and pure sliding window protocol is used, then what is sender window size:

(a) 61                       (b) 41                         (c) 21                          (d) 11

- From the above question no. 3, how many bits are required in the sequence number field?

(a) 6                         (b) 5                         (c) 4                               (d) 3

- What is the maximum size of data that the application layer can pass on to the TCP layer below?

(a) Any size       (b) 216 B to Header size        (c) 216 Bytes             (d) 1500 Bytes

- A building running CSMA – CD protocol is having a bandwidth of 512Mbps and distance of 2 kilometer’s then determine the minimum data size in order to detect a collision. Assume that the signal speed is 2,00,000km/s

(a) 1000 bytes       (b) 1250 bytes       (c) 1216 bytes             (d) 1280 bytes

- A system uses the Sliding window Protocol is having a bandwidth of 10Mbps with a window size of 100. What is the size of data if the distance between the sender and receiver is 72000 km and the propagation speed is 3 x 108 m/sec? Given utilization is 0.5

(a) 2048 bytes       (b) 4096 bytes        (c) 3015 bytes             (d) 3072 bytes

- A 40 Mbps broadcast network that controls medium access using polling has 20 hosts and time required for polling the next host is 80 μsec. Whenever a node is polled, it is allowed to transmit 4000bytes. Find the efficiency of the broadcast channel

A. 100/9       B. 100/11               C. 80/7                     D. 10/11

- In Ethernet, when Manchester Encoding is used, the bit rate is

(a) Half the Baud Rate (b) Twice the Baud Rate   (c) Same as Baud Rate (d) None

- Station A needs to send a message consisting of 15 packets to station ‘B’ using a sling window (window size 4) and go-back-N error control strategy. All packets are ready and immediately available for transmission. If every 6th packet that ‘A’ transmits gets lost (but no Acks from ‘B’ ever gets lost), then what is the number of packets that ‘A’ will transmit for sending the message to ‘B’ ?

(a) 29           (b) 33                 (c) 27                  (d). 25

Part B: Answer any two questions                                                                    [5X2= 10 Marks]

- Consider a Metro-Politian Area Network (MAN) with average source and destination is 20 KM apart and one way delay of 100 micro sec. At what data rate does the round trip delay equals to transmission delay for a 1 KB packet?

- Compute the fraction of the BW that is wasted on overhead (header and retransmission) on a heavily loaded 50Kbps satellite channel with data frames consisting of 40 header and 3960 data bits. ACK frames never occur. NAK frames are 40 bits. The error rate for data transmission is 1%, and error rates for NAK frames are negligible. Assume number of packets are 100.

- What is the requirement of layering structure of network?

Part C: Answer any two questions                                                                    [5X2= 10 Marks]

- What is fixed framing and variable length framing? Explain the pros and cons of each method in detail.

- Illustrate Routing of Packets within Virtual Circuit Subnet.

- Write the advantages of optical fiber over twisted and coaxial cables.